PCI - The PCI System information & Exploration tool.

The following is a somewhat rambling text, from which you should be able to

extract everything you ever wanted to know about this program! Please excuse my

poor documentation style; I really hate writing the docs :-) So read

everything, the answer's probably there, somewhere.

� General

This code is Written by Craig Hart in 1996-2002 and is under constant

development. It is released as freeware; please use and modify at will. No

guarantees are made or implied. Commercial use is also specifically permitted,

without restriction. It's free, use it!

I'd appreciate credit if you find this code useful. I'd also be interested to

see any code you may develop or modifications you create to this code...

suggestions & bug fixes are _always_ welcomed.

I can be reached by email: chart@hyperlink.net.au

See my home page for the latest version of all my software releases,

programmers information, updates for PCIDEVS.TXT and much more:

http://members.hyperlink.net.au/~chart

NOTE: Wherever you see the term PCI in this document, you can substitute AGP

and/or CardBus, if appropriate, instead. PCI is the "root technology" upon

which AGP and Cardbus are built, and share a common saet of basic design

standards. AGP is pretty much just a new physical and electrical form of the

PCI bus; CardBus is the 32-bit "version" of PCMCIA; software-wise, they're

virtually identical. This also covers all the 'other' PCI variants, such as

SmallPCI, PCI-X, etc.

� Why create this program, anyhow? What use is it? What does it do?

PCI basically produces a report of the PCI, AGP & CardBus devices fitted to a

PC, including the system chipset. A plethoria of information is reported on,

including system resource useage (IRQs, Memory ranges, etc), capabilities

(busmastering, caching), setup data (device latencies, general capabilities,

features, subsystem info), and much, much more. A text-file PCIDEVS.TXT lists

thousands of known vendors, devices and subsystems, which PCI will refer to and

display the info from. PCI covers all PCI device derivitives, including PCI

64-bit & 66MHz options, AGP (all speeds to 8x), CompactPCI, CardBus and PCI-X.

PCIDEVS.TXT is a plain text file, so you can update it yourself, however it is

updated regularly (virtually daily) by the author, and the latest revision is

always available as a free download from the webpage (see general section).

This program was originally written purely for me to learn how to program the

PCI BIOS found in newer PC's. Since then, this program has proven to be vastly

more useful, especially in the hands of a technician, system builder, and also

in the hands of those about to purchase or upgrade a PC.

To a technician or system builder, PCI can act as a system information and

diagnostic tool. It lists the resources devices have been assigned (for

example, IRQ's, Memory regions, etc) so it is handy for finding conflicts.

Because PCI also identifies the devices fitted, it is handy in figuring out

_exactly_ what drivers are required when setting up a system.

PC buyers don't need to open a PC to see exactly what they're getting: your

vendor can't tell you fibs about the chipset, graphic-card, or whatever, and

since the PC doesn't need to be opened to check what you ordered is what you

got fitted, you aren't loosing any warranties.

Second hand parts hoarders can figure out what they have, just by plugging it

in and running PCI - no more scratching your head over a mystery card you just

stumbled across amongst your latest piles of aquired junk. As above, finding

drivers becomes much easier when you can say *exactly* what brand model, and

revision of card you have.

To a programmer, PCI is a learning tool, since it's full source code is

supplied, and the dump configuration-space feature will help a programmer

discover how a driver alters the hardware to activate special features or

generally work with a given device.

� Using PCI

Just run it. Read the output. Be happy!

You must place PCI.EXE and PCIDEVS.TXT together in the same directory. You must

be "in" that directory before running PCI; in other words, do NOT run PCI from

a path. This is because PCI only looks in the CURRENT directory for it's data

file.

PCI (as of version 0.41�) will pause at the end of each screenfull to allow you

to read the report; press any key for the next page. Pausing is disabled if you

are using I/O redirection.

PCI's output can be redirected (using MS-DOS pipes), for example to a text file

on disk or a printer. IE "C:\>PCI > LOG.TXT" will generate a file on disk

called LOG.TXT  This is handy if you want to keep a perminant record of the

system under test, print it out, email it, or whatever.

PCI will be slow to generate it's report if run from a plain DOS computer from

a floppy disk drive (due to reading the rather large pcidevs.txt file). For

best speed run from within windows and/or from a hard disk drive. This is

because the data file is more than 250k in size and is therefore not cached in

memory. To improve speed under DOS a little, put buffers=20 in your config.sys

file and reboot.

PCI does not function under Windows 2000 or NT or XP. 3.x/95/98/ME work OK, as

does OS/2 Warp.  See the bugs and OS/2 sections for more info.

Syntax: PCI [/H] [/D] [/S] [/T] [/B] [/P] [/I] [/?]           ([] = optional)

Commandline parameters:

/H   Use direct-hardware access to retrieve the information. Normally, the

system BIOS is called on to retrieve the information, however there are some

BIOS's circa 1995 (Award v4.51 on Intel 430FX Chipset, early PCI Compaq's using

the Triflex Bridge chip) that incorrectly report some information. By bypassing

the BIOS, we can get the correct information. This only works if the BIOS uses

configuration mechanism 1. Mechanism 2 systems can use the BIOS method only.

Mechanism 2 systems are virtually non-existant as version 2.1 of the PCI

specifications insists on the use of mechanism 1 only... the number of

mechanism 2 chipsets is very small, and are now more than 7 years old.

/D   Dump the PCI configuration space for each device. This option generates

a 256 byte hex-dump of the PCI configuration space for each detected device.

This is handy for people trying to learn to program a device, for those looking

to discover any 'extra' undocumented registers in a device, observe the changes

made by setup or driver software, and also to fault-find this program :-)

/S   Produce summary report only. Lists vendors, devices, subsystem ID's

and IRQ's only. Usefull for when you need a "quick glance" and don't want to

wade through mountains of technical info. Still displays subsystem and IRQ

info, as these are typically the most-used features of PCI.

/T    Disble the BIOS IRQ Routing table tests. May be usefull if PCI crashes

during this test, or you don't want to clutter up a device report with this

'extra' info.

/B    List Bus, Device and Function info for each device. The bus number

tells you which PCI bus the device resides on (look at the PCI bridges to see

which bus number is 'created' by which bridge). The Device number is the PCI

device identifier for that device on that bus (there can be 32 devices on each

bus, and device numbers are generally non-contiguous). The function number is

the internal sub-funtion of that PCI CHIP (many devices are single-function and

only have a valid function 0, whilst others are multi-function and contain

several sub-functions; each device may have as many as 8 sub-functions).

/P    Read and display PCI IRQ-router information. The IRQ router is the

device that connects the PCI slots INTA-INTD lines to the sytem's 16 IRQ lines.

The first part of this option shows what IRQ's are *potentially* available for

the BIOS to connect each PCI slot. If you see Slot 00, it's an integrated

(built on the motherboard) device, not a physical slot. You can work out which

slot is which by using the /B command line parameter to display the Device &

Bus info for each card, and then match this info up by looking physically at

which card is in which slot.  Typically slots of the same bus are physically

laid out in numeric order, either left to right, or right to left; different

busses may be ordered differently to each other. Typically, all slots of a

given bus are physically next to each other rather than bieng randomly

distributed.

The second part of this option displays a table of slots and INTA-D link

values. To interpret this table, first note that a non-zero link value under

INTx means that that INTx pin on that PCI socket is wired to the IRQ router's

<number xx> input. A 00 means that INT line is not connected *at all*.

Let's look at a typical table:

SLOT BUS DEV   INTA INTB INTC INTD

01   0  17     01   02   03   05

02   0  18     02   03   05   01

03   0  19     03   05   01   02

04   0  20     05   01   02   03

00   0   1     01   02   03   05

What this table tells us is that Slot 01 and Slot 00 share the SAME link

values, whilst the other 3 sockets each have different link values (under each

INT line). How is this important? Each *link value* is mapped to a system IRQ,

(but only if the PCI card signals that it requires an IRQ).

If Slot 00 and slot 01 are both populated (with cards that request an IRQ on

the same INT line), they will both be assigned the SAME IRQ! similarly, if a

card using INTD in Slot 2 and a card using INTB in slot 4 both require IRQ's,

they will each be configured to the same IRQ. Depending on your OS, this may or

may not be a good thing - some OS's don't support shared IRQs in this fashion

(A.K.A.  the dreaded IRQ conflict) You may also want to know if you're going to

get an IRQ conflict if you're about to fit a new card and don't want to use

"trial and error" to get a non-shared IRQ assignment.

This table also highlights the other interesting points - if two slots have

different link values under the same INT line, they CANNOT be configured to the

SAME IRQ! Each slot will always be assigned a different IRQ from each other.

Also, once a link-value has been assigned an IRQ, all other slots where that

link value appears automatically take on that IRQ, so in many (but not all)

cases you can predict what IRQ you will get when you fit a card to that slot.

This sort of prediction an help in, for example, bulk-loading pre-configured

software images into mass-produced systems.

If you have IRQ assignment problems, these tables may highlight why - perhaps a

certain slot is incapable of having a desired IRQ assigned, or perhaps a given

slot has a certain INTx line not connected. Careful examination of these tables

will tell you straight away if this is the case or not.

The only thing this table won't tell you is which IRQ the BIOS will actually

assign each slot - there is no way to determine this and most BIOS's seem to

have no logical sequence for assigning resources; you get what you get! At

least PCI will tell you what the BIOS has done, and it's up to you to decide if

it's what you want, or not.

For more info, see also "PCI System Architecture" by Mindshare, inc. Chapter 11

has several diagrams (figures 11-4 and 11-5 in the third edition, pp216-217) of

IRQ router implementations that make this a lot easier to understand!

Finally, don't worry about the actual numerical link value (except for 00, of

course) - that only has meaning to the BIOS; all we're looking for is the

patterns the numbers represent, the numbers themselves are meaningless;

letters, cute pictures, roman numerals, colours, whatever! would serve the same

purpose just as well.

/I   Installer mode. Special mode for use with external programs to do

unattended driver installations and the like. Documented sperately below. Read

on.

� A note on PCMCIA and CardBus Support

PCI does NOT implement PCMCIA device detection!!  PCI can only detect CardBus

cards that are inserted into CardBus controllers.  PCI can NOT detect PCMCIA

cards, legacy-type PCMCIA controllers, or CardBus cards that are plugged into

PCMCIA controllers. PCMCIA is a 16-bit standard, based on the ISA bus

standards, and has nothing to do with PCI. CardBus is a 32-bit "version" or

"upgrade" if you like, of PCMCIA; it uses the same physical connector and has

backwards compatability for PCMCIA cards, but it appears to system software as

just another PCI bus.

It seems that some sort of CardBus driver *may* also need to be loaded for

CardBus detection to work. CardBus Detection seems to work reliably under

Win9x/ME ONLY if the PC Card (PCMCIA) icon is visible in the taskbar, and

reports the PC card as recognised and present in the socket. This may be

somewhat counter-productive for those using installer mode! In the end, it

comes down to what sort of CardBus support the BIOS has; if the BIOS supports

booting from a CardBus card, for example, your chances are pretty good...but

most BIOS's don't have CardBus support.

� PCI Crashes the Computer!

It has come to my attention that there are VERY SMALL number of PCI devices out

there on the market which are intolerant to having their configuration space

registers read by software other than their own driver. This means that

programs like PCI upset these devices when they read the device's configuration

space, and the net result is a lock-up, crash, GPF or "blue screen of death"

when PCI scans that device.

In order to "work around" this buggy hardware, PCI during installer mode and

summary mode only reads the "standard" 64 configuration space registers, not

the whole 256 registers. The first 64 registers give us the basic data which

installer and summary modes need, and should not upset the device.

So, in summary, try PCI -S or PCI -I and see if the crash doesn't happen. If

PCI works OK in that mode, then that is all you can use on that hardware. There

is no way to run PCI in it's "full" reporting modes on this sort of buggy

hardware - the full modes need to read the whole config space to get all the

data to create the report!

This is yet another example of vendors doing a poor job with their hardware

design processes, as this sort of bug just should not exist.

� What does "Known Bad Subsystem ID" mean?

Sometimes when running PCI, you'll see the following message: "Subsystem Vendor

XXXXh Known Bad Subsystem ID - no Vendor ID Available". This means that the

device has reported a subsystem ID which is not compliant with the PCI

standard, which PCI has taken special note of.

This is not an error or problem, and you don't have to report anything to me.

This is just PCI's way of letting you know that it has detected a non-complaint

subsystem ID (Which is listed in PCIDEVS.TXT as a type X entry - see the

section on PCIDEVS.TXT file format for further information).

Normally, the subsystem ID consists of the subsystem vendor ID (SVID) and the

subsystem device ID (SDID). The SVID lets you know what brand the device is,

and the SDID the exact device model. When the subsystem "system" was first

introduced, some vendors misinterpreted the standard, and failed to insert

their SVID properly. Vendors soon corrected their errors, but there are at

least 38 known "erronous" devices out there on the market, possibly the best

known of them is the VIA 82cx86 southbridge-series USB Controller.

With a non-standard SVID, you can't tell what vendor made the device, so

instead of reporting "unknown vendor" or identifying the incorrect vendor, we

simply point out the fact that the vendor can't be determined.

� What's the ROM IRQ routing table and IRQ-router info?

The PCI ROM IRQ routing table is a scheme devised by Microsoft to assist

Windows Device Manager to reprogram PCI IRQs. It falls under the category of

'plug and play' support and is pretty much a vital component of any motherboard

these days.

Normally, PCI IRQ's are set by the BIOS at boot time, and cannot be changed

after the system is booted.

Microsoft didn't like that and so they devised a mechanism to allows Windows to

change the IRQ assignments after boot-time. This makes sense, since Windows is

much more capable of handling scenarios like hot-docking (and the shuffling of

resources that this creates), that the BIOS clearly can't cope with in real

time or hope to forsee at boot time.

The IRQ-router info is a PCI BIOS call that predates the ROM IRQ routing table,

but offers similar information. The two should always agree! The disadvantage

of the BIOS call is primarily that it is slow, but also that it is based on the

very earliest versions of the PCI specifications, and thus lacks the

expandability and flexability of the newer ROM routing table for future

growth.

The PCI standards say that the hardware vendors are free to connect PCI INT

lines to system IRQ's in any fashion they like; there is no "standard". In

practice, there are at least 6 different ways chipsets have actually

implemented INT-to-IRQ connections. Since this is the case, Windows needs to

know a few things such as which INT's can be routed to which IRQs, which

INT-to-IRQ mappings are fixed, and which INTs belong exclusively to PCI. With

that data in hand, Windows can re-configure IRQs for PCI devices when/if

required to resolve resource conflicts (in conjunction with all the other

system-wide resource data that Device Manager holds), because the user has

requested a specific resource setting, because of a hot-dock event, CardBus PC

Card event, or whatever.

Only win95B (OSR2) or later supports this standard - Win95 original and OSR1

don't.... under 95 & 95A, PCI IRQ resources are fixed (Windows uses whatever

the BIOS assigned at boot time, and can't alter them) which is yet another

reason not to use 95A, if you still do! 95A is particularly lousy at managing

any sort of dockable/CardBus equipped laptop. 95B also has a few quirks, and is

increasingly showing limitations on the most modern chipsets; therefore it is

strongly recommended that Win98SE, Win2000 or newer OS's are used on modern

(P4, Athlon XP, etc) hardware.

The knowledge that the routing table(s) are present and valid is therefore

essential for determining why Windows is or is not correctly handling PCI IRQ

resoures. All modern (PIII+) motherboards should support the ROM routing table

- but many (PII and earlier) don't, or the table is faulty; PCI therefore is

able to tell us at a glance, wether IRQ management within windows is possible

on that motherboard, and wether the ROM table is in working order or not. I

strongly suggest you don't invest in any motherboard that has a non-existent or

faulty implementation of the ROM IRQ routing table.  Needless to say, a

dockable and/or CardBus equipped laptop without a working routing table is

going to be nothing but trouble.

Many older motherbards can gain (or fix bugs in) routing table support with a

FlashBIOS upgrade - visit your motherboard vendor's website (or

http://www.wimsbios.com) for the latest FlashBIOS and see if that helps.

Complain loudly to any vendor with a buggy routing table; with the advent of

Win2000, and the growing popularity of dockable laptops, this sort of thing is

going to be an absolute minimum requirement (along with ACPI support) for a

true overall 'plug and play' operating environment.

� Expansion ROM Information

PCI will report on any card that has a ROM *SOCKET* fitted (IE SCSI, Video,

Ethernet cards). PCI will tell you want size ROM is supported in the socket (ie

64k, 256k, etc). The one thing PCI can't actually tell you is if there really

is a ROM fitted, or not! Many ethernet cards have a ROM socket for a network

bootROM, for example, but the majority of cards don't actually have a bootROM

fitted.

The reason why PCI can't detect the ROM is that the programmer is supposed to

enable the ROM to become visible within the PC's address space; then the ROM's

signature and checksum bytes can be inspected to validate it's physical

presence.

To enable the ROM you need to decide where in the system's memory map to put

it. This is of course where the problm lies. With modern OS's operating in

protected mode, you can't just 'stick it anywhere' as the OS may be using that

address space already, and with virtualisation of the address space for V86

tasks, any memory accesses we make probably won't wind up going the the right

physical location anyhow. To make this work I would have to interact directly

with the memory manager to request an 'empty' physical memory block, request

the logical-to-physical address mapping data, then map the ROM into the given

region. IMHO this is well beyond my skills, so I gave up!

The ROM's aren't always enabled; at BIOS POST-time, they're enabled, copied to

Write-Protected RAM, then disabled again. The RAM-image is made to appear in

the UMB space (IE between 640k and 1Mb), just like a 'normal' ISA option ROM.

It's a bit like ROM BIOS Shadowing, if you like.  The ROM stays disabled

forever more, since the copy in RAM becomes the 'live' copy.  This is done for

speed (BIOS ROM's were shadowed for speed back in the '286 days!), but also it

is also done because the PCI bus usually decodes into memory adress ranges well

above 1Mb.

� Installer mode

New to version 0.41� is installer mode. Installer mode overrides all other

commandline parameters, except /H. Installer mode simply produces a plain dump

of all the 'technical' data about the PCI devices, but doesn't scan

PCIDEVS.TXT, doesn't draw fancy colours, doesn't mess you about in general.

The purpose of this mode is for use with automated systems that set up new

computers. In conjunction with some batch files or a second program, you can

take PCI's output and use it to select the right set of drivers to include in

the build of a new system. Because all the vital slot data is included, the

drivers can also be pre-configured to load properly with the right IRQ already

selected, etc.

Here is a sample of installer mode's output:

V:1106 D:0598 S:00000000 B:0 E:00 F:0 I:00 N:- C:06 U:00 P:00

V:1106 D:8598 S:00000000 B:0 E:01 F:0 I:00 N:- C:06 U:04 P:00

V:1106 D:0586 S:00001106 B:0 E:07 F:0 I:00 N:- C:06 U:01 P:00

V:1106 D:0571 S:00000000 B:0 E:07 F:1 I:00 N:- C:01 U:01 P:8A

V:1106 D:3038 S:12340925 B:0 E:07 F:2 I:11 N:D C:0C U:03 P:00

V:1106 D:3040 S:00000000 B:0 E:07 F:3 I:00 N:- C:06 U:00 P:00

V:121A D:0003 S:0017139C B:0 E:17 F:0 I:10 N:A C:03 U:00 P:00

V:1011 D:0014 S:01001186 B:0 E:19 F:0 I:09 N:A C:02 U:00 P:00

Note the absence of the normal header and copyright information. This makes

writing a filter easier as you don't have to find that start of the data.

Leading zeros are inserted to make sure that the entries always start a fixed

number of colums from the left. This means searching the results is MUCH easier

for software.

PCIDEVS.TXT isn't processed and isn't required. This means that you can save on

diskspace and also that the program runs much faster from floppy disk; it's

virtualy instant to run no matter how complex your device list.

What's reported?

V: 4 digit [V]endor ID (Hexadecimal)

D: 4 digit [D]evice ID (Hexadecimal)

S: 8 digit [S]ubSystem ID (Hexadecimal), or 00000000 if no subsystem ID

B: PCI [B]us number

E: PCI d[E]vice ID number

F: PCI device [F]unction Number

I: [I]RQ number, or 00 if none

N: I[N]T assignment, or "-" if none

C: PCI [C]lass code (Hexadecimal)		(Added v0.42�)

U: PCI s[U]bclass code (Hexadecimal)		(Added v0.42�)

P: PCI [P]rogramming interface (Hexadecimal)	(Added v0.42�)

** More fields may be added onto the end of each line, if deemed required in

future. Write your filter to cope!

Installer mode may struggle with or ignore CardBus devices - see the cardbus

section for more info; but basically, if your BIOS doesn't have full Cardbus

support, neither will installer mode. Sorry! There's nothing I can do -

complain to your vendor for better BIOS CardBus suport!

� OS/2 Warp

Firstly, I don't own or have access to OS/2, so the the following is only

heresay as reported by OS/2 users; but I believe it to be accurate. PCI works

OK under OS/2 - it just runs in a DOS box like any other legacy DOS

application.

Apparently an OS/2 specific port of PCI is now available, which is not written

or supported by me! It is my program, heavily modified to be a native OS/2

console application, as well as a few minor behavioural changes to suit the

author's personal taste.  The ported versions carry the letters VK on the end

of the version number - I.E. Version 0.45�VK.

For questions regarding the *vk version, please mail Veit.Kannegieser@gmx.de

His website has an even longer URL than mine; the English version of which

seems to be: http://www-user.tu-cottbus.de/~kannegv/index_e.htm

I encourage OS/2 users to try both versions.  Veit's port usually lags a

version number or two behind mine, and I've had odd reports of his version

crashing where mine doesn't, however there's no reason not to use Veit's

version instead of mine if it works better for you.

Generally, OS/2's AGP support in OS/2 is minimal, at best. Whilst there is

nothing stopping AGP working in OS/2, very few drivers bother to support it at

all, or only partially (i.e. basic PCI level support, or 1x speed only). You

have to ask yourself if you need AGP under OS/2 anyhow - unless you're somehow

doing 3D gaming, animation or texture mapping, AGP really won't make much of a

difference. Plain old PCI-speed is more than good enough for general 2D work.

OS/2 supports PCI interrupt sharing, but many OS/2 drivers (about half) don't.

Thus, it's usually a requirement that there be no shared IRQ's in an OS/2

system. This is getting increasingly awkward to achieve on modern chipsets and

notebooks, further restricting OS/2's continued growth.

� Does my AGP 1x/2x/4x support work, or what?

There seems to be some misinformation about support for the faster AGP speeds

out there, not the least of which was caused by an article I wrote some years

back about PCI chipsets on AGP PCB's (cards sold as 'AGP' type, but really just

have 'PCI' level features).

Here are the facts. Don't trust anything you read except the following as

there is a lot of misinformation or just plain outdated commentry out there on

the web...

All AGP cards made since 1998 are the true AGP type; that is they support

enhanced speeds beyond that of PCI (I know of no exceptions to that rule!).

Only a small number of very early AGP cards are really just PCI chipsets on AGP

cards, but be aware they do exist (mostly in older systems circa 1995-7).

To identify a "true" AGP card, check for the New Capabilties List of the device

for an "AGP Capabilty" entry.  AGP capability version 1.0 supports AGP 1x and

optionally 2x, whilst version 2.0 adds 4x support and version 3.0 adds 8x

support. All AGP cards MUST have this 'New Capability' data present to operate

in AGP mode.

Next, check your chipset's AGP bridge's New Capabilities List to determine the

chipset's AGP support level. If you don't have an AGP Bridge somewhere in your

system, you probably don't have an AGP video card :-)

Compare the two AGP Capability Lists and identify the highest common supported

speed. For example, the Nvidia GeForce2 MX has a maximum AGP speed of 4x, but

the VIA MVP3 AGP bridge has a maximum speed of 2x, so 2x is the fastest

possible speed available on that system. In this case, the GeForce2 MX will

perform more slowly than if it were in a more modern system and able to run at

it's full 4x speed.

Next, see if AGP mode is actually enabled (It's written as a NO in red if it

isn't and as a YES in green if it is), and read off the selected speed.

Note: If you run PCI *WITHOUT* your motherboard's AGP drivers, *AND* the

display card's drivers correctly loaded and functioning, AGP WILL REPORT AS

DISABLED!!!! That means running PCI from plain DOS will ALWAYS report AGP as

disabled, no matter what! Always make sure both the chipset and video drivers

specific to your chipset and display adapter are correctly installed and

functioning, or AGP support will not work!

To get AGP mode enabled, the host CHIPSET must be initialised into AGP mode (IE

the GART must be configured and enabled) and then the display adapter must be

put into AGP mode and accessed that way. AGP mode is not compatible with legacy

type VGA adapters, and thus it won't work when your OS is trying to access the

VGA card the 'traditional' way.

SO.. the only way to properly check on AGP support is to run PCI in a DOS

window under your operating system... this means Win2k/NT/XP machines can't be

checked currently. Sorry! (I'm working on it!!)

� Bugs, stuff yet to come, limitations, etc.

PCI is known to not operate under Windows NT, 2000 or XP. If you run

NT/2000/XP, you must boot with DOS or Win95/98/ME to run PCI. This is because

NT's HAL denies PCI access to the system BIOS and I/O ports necssary for PCI to

operate. I'm working on a version that will work on newer OS's, but it's a hard

task for me - please be patient while I learn about win32 coding, device

drivers and the like.

Basically, full decoding of all the PCI data is yet to be implemented... mainly

because I don't own full copies of all the 'official' PCI specification

documents, (And I am unable to get them as they cost $$$ and I don't live in

the USA so I have no easy way to transmit payment...presuming I wanted to pay

for them at all). So, I am reliant on snippets of information collated from

books, data sheets, YOUR INPUT, etc etc. See the web page for more info, and

the text of the next section.

I now have a copy of most of the specs circa PCI 2.2 level (Thanks to a

generous [you know who you are] who posted them at his expense all the way from

Canada to Australia for me!), but PCI 3.0 is just about to come out so again I

will be "behind" shortly.

Having said all that, PCI is now a very mature product (developed now for ~7

years) and it is 'virtually' complete. There isn't much left to do, and each

new version is improving support, for example CardBus support is now finally

working, and AGP 8x support has recently been added.

� And to the PCI SIG:

Official PCI Programming information is not freely available. The controlling

body the "PCI Special Interest Group" has determined that documentation will be

made available only to those willing to pay for it. And it's not cheap, or

easily obtainable outside the USA. I have the following to say to the PCI SIG:

Charging money for your documentation not only restricts programmers and end

users from adopting the PCI standard, but makes a mockery of the concept of

open information sharing that the Internet has always stood for. Rather than

take such a petty attitude towards Joe Average programmer, why not release your

documentation freely in electronic format? It can only help establish your

standards further.

Freely releasing formal documentation would also mean that programmers such as

myself can stop guessing about PCI, and stop writing up alternative

documentation that may, ultimately, be incorrect. I'm sure that the current

host of "third party" documentation (along with it's errors) can only make PCI

look BAD, not good.

Remember MCA? In 1987 we already had what PCI now offers.. please don't kill

PCI the same way IBM killed MCA.

� How to "parse" PCIDEVS.TXT

Writing your own code to use my database? GREAT! You're more than welcome: the

database is freeware and you may use it for any purpose, including commercial

purposes. Here's how *I* parse the list, in "pseudocode". I hope this makes it

easier for you to write a parser too. Please forgive me if you think this is

poorly designed/could be done better; the technique stems from three things:

- Sourcefile is TEXT, not Binary, so there are no fixed field lengths.

- Many fields are optional so the catch-22 of "you dunno if it's there 'till

you read it, but once you've read it, you can't go back a line if it wasn't

what you expected" applies.

- The program has been added to and expanded "ad hoc" over a 5 year period.

Inevitable oversights and extensions have been dealt with several times.

- The basic file format can't be changed since there are many programs

(including a couple of commercial packages) relying on my file format not

changing.

Open the file as text (not binary) and read top to bottom, one line at a time.

1. Look for V entries, until you make a match to the vendor ID, or EOF in which

case report unknown vendor & device. Display Vendor Name.

<the file position pointer now points to the list of devices for that Vendor>

2. Read D entries until you get a match to the device ID, or hit another V

entry or EOF in which case stop and report unknown device. DONT display Device

Name yet!!

<the file position pointer now points to possible device revision records>

3. Read R entries until you get a match with the device's revision ID, or you

run out of R entries, or EOF. (There may be no R entries at all). If you match

R, then display that entry for the Device Name, otherwise display the Device

Name from the D entry... R is more accurate than D, but you might not match R.

<the file position pointer now points to possible subsystem records>

4. If you matched D(and/or R), and the subsystem ID is <>00000000 then (look

for subsystem match):

in a loop, read a line, and

- try to match an X code with the 8 digit subsystem ID.

- try to match an O code with your subsystem vendor ID. Remember the

"generic" part description. Note the match, Keep going in this loop.

- try to match an S code with your subsystem device ID, but only if the O

code has already been matched.

exit when you matched an X code, or (matched an S AND an O code), EOF, or

you hit another V or D code.

- if you EOF or matched neither X, O nor S, report unknown subsystem ID.

- if you matched an X entry, report info next to X entry, and warn user this

is a known "oddball" device that has an otherwise invalid subsystem ID. (Ignore

any partial S-but-not-O match, if any - its a false alarm).

- if you matched O BUT NOT S, report the "generic" ID you remembered. Warn

user it may not be "fully" accurate, but is just a guestimation.

- if you matched O AND S, report info next to S entry; you exactly matched the

subsystem ID.

5. close list

6. If you have an O code, re-open the list. Scan list reading the V entries,

but try to match with the O code. This tells you the OEM name. Stop at match or

EOF. If EOF, report unknown OEM ID. Close list.

List must be closed and re-opened because the O name may be "higher" up the

list, thus a scan-to-the-end of the list will not match. This also means this

check must be done last since it causes us to "loose our spot" in the list.

Filepos() doesn't work (in Pascal, at least) since we're working with a

textfile!! (Argh!!)

If you find an invalid code letter, IGNORE IT; just move onto the next line.

This lets us add new code letters and not 'break' existing code.

� Other formatting notes for PCIDEVS.TXT:

All useful lines are formatted thus:

<Code Letter><Tab><2-8 hex digits><tab><text>

You may NOT replace <tab> with <space>! Also, do NOT replace a tab with eight

spaces!!! the parser counts characters left to right, and looks for the tab

character, so wrong, extra or missing characters will result in a wrongly

parsed line. This means the file formatting must be kept strictly in check at

all times. Use my CHKPCI utility (Available seperately from the website) to

inspect and validate your changes.

Overall, total line length must be 255 characters or less (Pascal language

limitation). Try to keep the text under about 70 chars for display legibilty

(excessive display wordwrap really is in poor taste :-/)

All entries must always follow numerical order, lowest to highest. This makes

duplicates almost impossible when editing, but the parser doesn't actually

care, since it works on "keep looking until you run out" principle. A "tiny"

efficiency could be added by coding in "if database ID > our ID, give up" but I

hardly see the point, since the program runs faster than you can blink anyhow.

You should ignore all lines not starting with a valid code letter,tab sequence.

This allows clarity by inserting blank lines and comments wherever it may

please you. For clarity, begin comment lines with a <;> character; Accidental

capitilisation of the first word of a comment could lead to a wrongly parsed

code.

Valid Codes:

V	Vendor ID. 4 digit hex number.

D	Device ID. 4 digit hex number.

R	Revision ID. 2 digit hex number.

X	Incorrectly formatted susbsystem ID. 8 digit hex number.

O	Subsystem OEM ID. 4 digit hex number. (top 16 bits of subsystem ID)

S	Subsystem device ID. 4 digit hex number. (low 16 bits of subsystem ID)

The codes must always appear in this order in the file. Multiple O and S

entries may appear. O entries may appear without S entries. Only V and D

entries are required to identify a device - all others are optional and my be

omitted.

A note on R entries: R is NOT permitted under a subsystem entry. A chipset

revision is just that - the BASE CHIPSET's revision. The OEM can't have any

influence on the chipset's revision, since he doesn't make it! Thus, R is of no

use under the subsystem. I very much doubt any OEM has made two different model

cards by carefully buying different revision chips from the chipset vendor!!

A note on X entries: X entries are very rare. In the "early days" of subsystem

ID's, some vendors apparently thought they had carte blanche to put in any

number they liked. WRONG! However a few devices now exist with nonsensical

subsystem ID's like "55555555" or "F0F01234" and suchlike. X entries take care

of these few "oddball" devices. I don't expect to add any more than one or two

new X entries, ever.

** New code letters may be added from time to time, so your code should always

ignore any unknown code letters. This lets up expand the scope of the file

without 'breaking' existing code.

� Program Revision Information

version 0.00� : Initial private release: Didn't crash my home PC!

version 0.10� : fixed bug whereby if either vendor byte is $FF, we would

skip the device. Ouch! Now finds devices reported as xxFF on

some buggy Award BIOS's chipsets. (Notibly, 80FF, not 8086

for the 82430FX IDE Ctrlr).

fixed problem decoding infotable[6].

fixed select timing decoding fixed irq=$ff decoding

version 0.11� : fixed bug displaying select timing modes

version 0.12� : added "subsystem id" information display

made cache line size display only if it exists (ie is<>0)

added /H parameter to use cfg mechanism 1, not BIOS to

read the hardware directly (gets around 80FF bug)

added bus mastering capability & latency reporting

added prefetchability reporting to memory ranges

fixed up several minor display and reporting issues

made IRQ display only if it's valid

version 0.13� : fixed some display formatting bugs

subsystem ID's only displayed if valid (<>00000000)

Added subsystem recognition system (after many requests)

Added ability to direct output to a file ie "C:\>pci > log"

version 0.14� : Added commandline parameter /D to dump device configuration

space, if required. This is usefull when programming a

PCI device, and/or looking for 'hidden' registers, features,

etc etc.

version 0.15� : added subsystem vendor display; it's not always accurate,

(Some vendors are ignoring the rules) but when it is right,

it can be very useful.

Added capabilities list decoding & display - not yet tested!

Tidied up the code here and there - fixing my cruddy coding

fixed major bugs with Expansion ROM info display

fixed minor potential bug with I/O port decoding

added interface type to class code line

version 0.16� : Fixed a stupid bug with new capabilities list display

Added raw-dump of Power Management capability data

version 0.17� : fixed some minor display formatting problems

fixed several problems decoding type 2 headers

fixed another bug with expansion ROM decoding

added some more decoding of type 1 headers

added some more decoding of type 2 headers

added info on PCI IDE controllers & mode(s) of operation

added error checking for case of VENDORS.TXT missing

added some decoding of "power management" new capability

version 0.18� : fixed display formatting bug with PCI IDE info

version 0.19� : added IRQ summary to end of display

added generic subsystem ID detection & note

fixed compiler leaving debug-info in EXE

cleaned up a few minor display issues

version 0.20� : added several new class codes intoduced with PCI spec v2.2

added decoding of command register

fixed "status 0200h ()" display formatting bug & redrew

the general layout of the Status info to avoid commom

wordwrap problem and to more correctly display timing +

busmaster info next to their "source" register

fixed "Number of PCI busses : 0"

version 0.21� : added MSI Capability detection

added shared IRQ summary

indented capabilities listings to make them more readable

fixed error in classes.pas missing out the last few entries

added ExpROM decode for type 1 headers

added I/O port range decode for PCI Bridges

fixed display wordwrap problems

full decoding of Self Test function byte

version 0.30� : converted to PCIDEVS.TXT format, added subsystem prediction

code

version 0.31� : fixed slight bug with unknown subsystem vendor ID

added /S parameter for brief summary only report

changed IRQ reporting to reject bad IRQ values (0<IRQ<16)

this fixes runtime error 201 on compaq triflex bridge!

version 0.32� : added revision ID decoding for more accurate chipset-id

added a load of info in the documentation on PCIDEVS.TXT

structure, parsing, etc

fixed problem with IRQ list wrong in summary mode because IDE

IRQs not recorded

Updated help to current feature set

version 0.33� : added PCI IRQ Routing table Win9x compatibility tests

version 0.34� : fixed some bugs in new routing table check routine

version 0.35� : fixed capability pointer bug with type 1 headers

IRQ summary display formatting fixed

PCI major revision number leading 0 dropped

help page again fits into one 25 line screen

added display bus/device/function info option

version 0.40� : added 'None' to "IRQ's dedicated to PCI" for when there are

truely none reported in the IRQ routing table info

added first attempt at PCI slot INTx routing data display

fixed a bug in "IRQ's dedicated to PCI" omitting IRQ 15

added latest PCI 2.2 class codes; fixed bugs in classes code

better error checking on AGP capability list

version 0.41�	: added all missing capability ID's per PCI rev 2.2 specs

fixed incorrect device name on slot irq-router info

added pause for keypress function when writing to screen

fixed new capabilities list bug for first capability = last

capability signature (i.e. no capabilities listed)!

fixed up ROM info - was totally wrong previously!

added /I installer mode report for system builders

/? now works even if PCIDEVS.TXT missing

help on error messages now more comprehensive

version 0.42� : fix up class code FFh reporting

added class, subclass & prog IF info to installer mode

version 0.43� : fix up errors in power management capability info

fix a spelling error -> "decryption"

don't read past config register $3f in installer and

summary modes to avoid crashing on intolerant hardware

fix some truncated text strings in class codes

added sanity checks for conflicting command line switches

added a lot to 'new capability' decoding section

fixed all the bugs with scanning cardbus devices

version 0.44� : fixed pci-x max speed 66, not 64

fixed shared IRQ count wrong if IDE controller IRQ field

reported by device

fixed duplicate scan of CardBus issue with some PCI BIOS's

added new capability code 09h recognition

fixed another CardBus bug with unconfigured controllers

version 0.45� : fixed overflow bug due to compiling with overflow checking

on. this has also reduced size of executable slightly

fixed potential bug with checksumming corrupt routing tables

extensively revised and expanded documentation with a load

of general comments and background info, tips etc.

fixed bug with parsing comment lines in middle of subsystem

entries in pcidevs.txt

version 0.46� : added support for AGP v3.0 as per rev 0.95 of AGP v3.0 specs

(This adds support for AGP 8x recognition)

further revisions, expansion and updates to documentation

version 0.47� : added revision field to Installer mode

added class code for EHCI USB controller

fixed some minor display indent-formatting issues

added more decoding of type 1 headers (PCI bridges)

added more decoding of PCI-X capability info

fixed a couple of minor code errors and a class code database

error

added support for PCI v2.3 standard

fixed more bugs in pci power management capability info

<EOF>